CSE 209 — Computer Architecture

Chapter 3 · Arithmetic for Computers (short slides)

Lesson notes · Exam approach · TF practice with solutions · Quick notes (Tushar Sir's portion)

## How to Read These Notes

| EXAM FOCUS | Exam importance, where it appears in the TF collection |
| --- | --- |
| MEMORIZE | Rules, tables and formulas to memorize |
| TRAP | Common mistakes and exam traps |
| EXAM-READY ANSWER | Model wording you can write in the exam |
| NOTE | Extra explanation / clarification |
| Grey box, monospace | Code, worked tables, derivations and hand-drawn sketches |
| Centered grey box | Formula to remember / final numerical result |
| Purple heading / ✔ green | Practice questions / their solutions |

## Contents

Exam Map — What the TF Papers Ask  —  arithmetic questions and how much they matter

Lesson 19 — Integer Addition, Subtraction and Overflow  —  overflow rules, the O/SA/SB/SR table

Lesson 20 — Multiplication Hardware  —  first and refined multiplier, flowcharts, step-by-step traces

Lesson 21 — IEEE 754 Floating-Point Representation  —  format, bias, ranges, special values, custom formats

Lesson 22 — Floating-Point Addition and the FP Adder  —  4 steps, flowchart, over/underflow, traced examples

Lesson 23 — Beyond the Short Slides (textbook insurance)  —  division, signed multiplication, MIPS FP code, guard/round/sticky, subword parallelism

Quick Notes — Arithmetic  —  merged revision notes + key diagrams

NOTE Slide numbers are PDF pages of Chapter_03_Arithmetic-shortslide.pdf (Tushar Sir, 20 slides; from PDF page 17 on the printed slide number is 2 higher). Lesson 23 covers textbook topics that are NOT on the short slides but are asked repeatedly (division even in 2022-23 CSE 209); those questions sit in the tracker's Outside Notes tab. All traces and encodings were generated and checked by _build/traces.py and verify.py.

# Exam Map — What the TF Papers Ask

Every Tushar-Sir-part question that this deck answers (★ = 2019-20 or later):

| Session (paper) | Q | What is asked | Marks | Lesson |
| --- | --- | --- | --- | --- |
| 2023-24 (CSE 209) ★ | 7(a) | Multiplication hardware: long-multiplication and parallel add/shift versions (32-bit); simulate 1000 × 1011 on each | 20 | L20 |
| 2023-24 (CSE 209) ★ | 7(b) | Purpose of a positive bias in IEEE 754; smallest and largest values in single and double precision | 15 | L21 |
| 2022-23 (CSE 209) ★ | 1(a) | Flowchart of floating-point addition; conditions for overflow, underflow, NaN and infinity | 10 | L22 |
| 2022-23 (CSE 209) ★ | 4 | Hardware block diagram + flowchart of the sequential and the refined multiplication algorithm | 15 | L20 |
| 2022-23 (CSE 305) ★ | 8(b) | 32-bit FP with 19-bit fraction: precision, smallest/largest absolute values, smallest denormal | 8 | L21 |
| 2022-23 (CSE 305) ★ | 8(c) | f1 < f2 (IEEE single): treated as unsigned, is i1 < i2? As signed, is j1 < j2? | 20 | L21 |
| 2021-22 (CSE 305) ★ | 6(c) | Block diagram of 32-bit combined multiply/divide hardware with a 32-bit ALU; functions of components | 10 | L20 |
| 2021-22 (CSE 305) ★ | 8(b) | Largest n such that every integer 1…n is exactly representable in IEEE single precision | 14 | L21 |
| 2020-21 (CSE 305) ★ | 13(a) | 32-bit FP with 24-bit fraction: why bias, value of bias, smallest/largest absolute values, smallest denormal | 16 | L21 |
| 2019-20 (CSE 305) ★ | 4(a) | 16-bit FP, largest 1.11…×2^63, smallest 1.00…×2^−62: format; −110.11₂×2¹; smallest denormal | 15 | L21 |
| 2019-20 (CSE 305) ★ | 4(b) | Why the first multiplier hardware is inefficient; changes; draw the efficient hardware | 12 | L20 |
| 2018-19 (CSE 305) | 5(a) | Hardware + flowchart of the sequential multiplication algorithm; flowchart of the refined version | 12 | L20 |
| 2018-19 (CSE 305) | 5(b) | 16-bit FP with 4-bit exponent: represent −1.25 and 0.375 | 8 | L21 |
| 2018-19 (CSE 305) | 5(c) | Decimal value of IEEE single 0xDF807500 | 5 | L21 |
| 2018-19 (CSE 305) | 7(c) | Use of bias in FP representation; encoding of infinity | 5 | L21 |
| 2018-19 (CSE 305) | 8(b) | Steps of FP addition with an example; overflow/underflow conditions | 10 | L22 |
| 2017-18 (CSE 305) | 5(b) | 16-bit half precision (4-bit exponent): convert 0.75 and 1.25 and add step by step | 15 | L22 |
| 2017-18 (CSE 305) | 5(c) | Decimal value of IEEE single 0x7F800005 | 5 | L21 |
| 2016-17 (CSE 305) | 7(b) | Simulate FP addition −0.25 + 0.625 on the FP adder hardware (fill the tables) | 15 | L22 |
| 2016-17 (CSE 305) | 8(a) | Overflow conditions in the O/SA/SB/SR table for add/subtract; Boolean function for V | 10 | L19 |
| 2016-17 (CSE 305) | 8(c) | Simulate unsigned multiplication 0110 × 0110 on the refined (shift-right product) hardware | 15 | L20 |
| 2015-16 (CSE 305) | 6(a) | Steps of floating-point addition with an example | 12 | L22 |
| 2015-16 (CSE 305) | 7(c) | Use of bias in FP representation; encoding of infinity | 10 | L21 |
| 2014-15 (CSE 305) | 8(a) | Overflow conditions in the O/SA/SB/SR table; Boolean function for V | 12 | L19 |
| 2014-15 (CSE 305) | 8(b) | Flowchart of the sequential multiplication; draw the register-optimized refined hardware | 13 | L20 |
| 2013-14 (CSE 305) | 3(a) | Steps for adding two binary floating-point numbers | 10 | L22 |
| 2013-14 (CSE 305) | 3(b) | IEEE 754 single-precision representation of −0.75 | 10 | L21 |
| 2013-14 (CSE 305) | 3(c) | Block diagram + flowchart for 32-bit unsigned multiplication with no separate multiplier register | 15 | L20 |
| 2012-13 (CSE 305) | 8(a) | Purpose of biased notation for FP exponents | 5 | L21 |
| 2011-12 (CSE 305) | 5(a) | Flow diagram of FP addition; overflow/underflow conditions | 10 | L22 |
| 2011-12 (CSE 305) | 6(a) | First and refined multiplication hardware; improvement of the refined version | 15 | L20 |
| 2022-23 (CSE 209) ★ | 1(b) | 6-bit ALU division: dividend 1011 1100 1111, divisor 110101 — registers at each step [Outside Notes] | 10 | L23 |
| 2022-23 (CSE 305) ★ | 8(a) | MIPS FP code: float a,b,c; double w,x,y,z; if (a+b>c) x = (y*z)/w; [Outside Notes] | 7 | L23 |
| 2021-22 (CSE 305) ★ | 5(c) | How can subword parallelism accelerate matrix multiplication / image processing? [Outside Notes] | 10 | L23 |
| 2020-21 (CSE 305) ★ | 10(c) | Briefly describe how to perform a signed multiplication [Outside Notes] | 5 | L23 |
| 2020-21 (CSE 305) ★ | 12(c) | Importance of guard digit, round digit and sticky bit when rounding [Outside Notes] | 9 | L23 |
| 2020-21 (CSE 305) ★ | 13(c) | MIPS FP code: float a,b,c; double w,x,y,z; if (a+b>c) x = (y*z)/w; [Outside Notes] | 7 | L23 |
| 2019-20 (CSE 305) ★ | 4(c) | MIPS FP code (double): if (a>b) c = c−(c*e)/d; else c = c+d; [Outside Notes] | 8 | L23 |
| 2018-19 (CSE 305) | 6(b) | 4-bit ALU division: dividend 11001001, divisor 1011 — registers at each step [Outside Notes] | 15 | L23 |
| 2017-18 (CSE 305) | 8(a) | 4-bit ALU division: dividend 11010101, divisor 1011 — registers at every step [Outside Notes] | 15 | L23 |
| 2017-18 (CSE 305) | 8(b) | MIPS FP code for float convert(float cels): ferh = 9.0*cels/5.0 + 32.0 [Outside Notes] | 10 | L23 |
| 2012-13 (CSE 305) | 8(b) | Flowchart of division for the given (refined) division hardware [Outside Notes] | 12 | L23 |

EXAM FOCUS Arithmetic is asked in every paper (155 marks since 2019-20 from the slides, plus the Lesson 23 rows). Three repeating types: (1) multiplication hardware — draw both versions, flowchart, trace (9 papers); (2) floating-point representation — custom 16/32-bit formats, bias, smallest/largest, hex ↔ decimal (13 rows); (3) FP addition steps / flowchart / traced example (7 papers). Division and MIPS FP code are not on the short slides but appear 4 + 4 times — do Lesson 23 too.

# Lesson 19 — Integer Addition, Subtraction and Overflow

Slides 2–4 (§3.1–3.2)

## 1. Addition (sl. 3)

Example 7 + 6 (carries in brackets): (0)(0)(1)(1)(0) carries … 0 0 1 1 1 7 … 0 0 1 1 0 6 … 0 1 1 0 1 13

## 2. Subtraction (sl. 4)

Subtraction adds the negation of the second operand: 7 − 6 = 7 + (−6) → 0000…0111 + 1111…1010 = 0000…0001.

## 3. Overflow Rules (sl. 3–4) ★

| Operation | Operand A | Operand B | Overflow if result sign is |
| --- | --- | --- | --- |
| A + B | ≥ 0 | ≥ 0 | 1 (negative) |
| A + B | < 0 | < 0 | 0 (non-negative) |
| A + B | opposite signs |  | never |
| A − B | ≥ 0 | < 0 | 1 |
| A − B | < 0 | ≥ 0 | 0 |
| A − B | same signs |  | never |

Overflow means the true result is outside the 2's-complement range −2n−1 … 2n−1 − 1. In hardware it is detected as V = Cn ⊕ Cn+1 (Lesson 9).

## Exam Approach

1.	Turn subtraction into addition of the negated B: A − B overflows exactly when A + (−B) would.

2.	Fill the sign table row by row; write V as a sum of minterms (no simplification needed).

## Practice — Lesson 19

(a) Actual — 2016-17 8(a) = 2014-15 8(a). (b) Constructed.

(a) The table lists O (0 = A + B, 1 = A − B), SA, SB (sign bits) and SR (sign of the result). Identify the overflow conditions and write V as a Boolean function. (10/12)

(b) 4-bit: does +5 + +4, −6 − +3, +5 − −2 overflow?

### ✔ Solution

| O | SA | SB | Effective operation | Overflow possible? | V |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | (+) + (+) | yes, if SR = 1 | SR |
| 0 | 0 | 1 | (+) + (−) | no | 0 |
| 0 | 1 | 0 | (−) + (+) | no | 0 |
| 0 | 1 | 1 | (−) + (−) | yes, if SR = 0 | SR' |
| 1 | 0 | 0 | (+) − (+) | no | 0 |
| 1 | 0 | 1 | (+) − (−) = (+) + (+) | yes, if SR = 1 | SR |
| 1 | 1 | 0 | (−) − (+) = (−) + (−) | yes, if SR = 0 | SR' |
| 1 | 1 | 1 | (−) − (−) | no | 0 |

V = O'·SA'·SB'·SR + O'·SA·SB·SR' + O·SA'·SB·SR + O·SA·SB'·SR'

NOTE The printed definition says 'V = 0: overflow occurred, 1: no overflow possible' — that is clearly swapped (V = 1 must mean overflow). State the convention you use; the function above has V = 1 for overflow.

(b) +5 + 4 = 0101 + 0100 = 1001: positive + positive gives sign 1 → overflow (9 > 7). −6 − 3 = 1010 + 1101 = (1)0111: negative + negative gives sign 0 → overflow (−9 < −8). +5 − (−2) = 0101 + 0010 = 0111 = +7 → no overflow.

# Lesson 20 — Multiplication Hardware

Slides 5–8 (§3.3)

## 1. Long Multiplication (sl. 5)

1000 multiplicand (8) × 1001 multiplier (9) 1000 0000 0000 1000 1001000 product (72) — length of product = sum of operand lengths

## 2. First Version of the Hardware (sl. 5–6) ★

Figure 20.1 — First multiplication algorithm (flowchart) and hardware (sl. 6)

64-bit Multiplicand register, initially the 32-bit multiplicand in the right half — shifts left each step.

32-bit Multiplier register — shifts right; its bit 0 is tested.

64-bit ALU and 64-bit Product register (initially 0).

Control test repeats 32 times: 1. if Multiplier0 = 1 add Multiplicand to Product; 2. shift Multiplicand left 1; 3. shift Multiplier right 1.

Start → Test Multiplier0 ──1──► 1a. Product = Product + Multiplicand ─┐ └──0────────────────────────────────────────────┤ 2. Shift the Multiplicand register left 1 bit ◄────────────────┘ 3. Shift the Multiplier register right 1 bit 32nd repetition? ── No (< 32) → back to Test ── Yes → Done

## 3. Optimized (Refined) Multiplier (sl. 7) ★

Figure 20.2 — Optimized multiplier: 32-bit multiplicand and ALU, 64-bit product shifting right (sl. 7)

Observations that make it efficient: half of the 64-bit multiplicand register was always 0 and only 32 bits are added each step; the product shifts right as the multiplier shifts right; the product's empty right half can hold the multiplier. So:

Multiplicand and ALU become 32 bits; the separate Multiplier register is removed — the multiplier is loaded into the right half of the 64-bit Product register.

Each step: if Product0 = 1, add Multiplicand to the left half of Product (the carry is kept); then shift the whole Product right 1 bit. Add and shift happen in parallel → one cycle per partial-product addition.

32 repetitions; the product is in the 64-bit register at the end.

Start → Test Product0 ──1──► 1a. Left half of Product = left half + Multiplicand ─┐ └──0───────────────────────────────────────────────────────┤ 2. Shift the Product register right 1 bit (carry enters MSB) ◄──────────┘ 32nd repetition? ── No → back to Test ── Yes → Done

EXAM-READY ANSWER Why the first hardware is inefficient (2019-20 4(b)): it needs a 64-bit ALU and a 64-bit multiplicand register although only 32 bits are ever added, a separate multiplier register, and three sequential steps (add, shift, shift) per bit → about 3 × 32 cycles. Changes: halve the multiplicand register and ALU to 32 bits, shift the product right instead of the multiplicand left, put the multiplier in the unused right half of the product register, and do the add and the shift in the same cycle → 32 cycles, less hardware (draw Fig. 20.2).

## 4. MIPS Multiplication (sl. 8)

Two 32-bit registers hold the product: HI (most-significant 32 bits) and LO (least-significant 32 bits).

mult rs, rt / multu rs, rt → 64-bit product in HI/LO (read with mfhi, mflo — textbook).

mul rd, rs, rt → least-significant 32 bits of the product into rd.

## 5. Combined Multiply/Divide Hardware (textbook, for 2021-22 6(c))

The refined multiplier and the refined divider (Lesson 23) have the same datapath, so one unit does both: a 32-bit Multiplicand/Divisor register, a 32-bit ALU (add for multiply, subtract for divide), a 64-bit Product/Remainder register that can shift right (multiply) and left (divide) and is written by the ALU into its left half, and a control unit that tests Product0 (multiply) or the sign of the ALU result (divide). MIPS exposes the 64-bit register as HI and LO: after mult HI:LO = product; after div LO = quotient, HI = remainder.

## Exam Approach

1.	Draw both diagrams with register widths (64/32) and shift directions.

2.	Flowchart: test, add, shift(s), count 32 (or n) repetitions.

3.	Trace table columns: Iteration | Step | Multiplier | Multiplicand | Product (first version) or Multiplicand | Product (refined). Use n-bit operands (n = 4 in exams).

## Practice — Lesson 20

(a) Actual — 2023-24 7(a). (b) Actual — 2016-17 8(c). (c) Methods — 2022-23 Q4, 2018-19 5(a), 2014-15 8(b), 2013-14 3(c), 2011-12 6(a), 2021-22 6(c).

(a) Draw the integer multiplication hardware (i) long-multiplication approach and (ii) parallel add/shift approach for 32-bit operands. Simulate each for 1000 (multiplicand) × 1011 (multiplier). (20)

(b) Using the refined hardware (32-bit multiplicand and ALU, 64-bit product shifting right), simulate unsigned 0110 × 0110. (15)

(c) Draw hardware + flowchart of the sequential version and of the refined version; for 2013-14 3(c) 'no separate multiplier register' = the refined version.

### ✔ Solution

(a) Diagrams: Fig. 20.1 (first) and Fig. 20.2 (refined). 4-bit traces (8 × 11 = 88 = 0101 1000):

| Iter | Step (first version) | Multiplier | Multiplicand | Product |
| --- | --- | --- | --- | --- |
| 0 | Initial values | 1011 | 00001000 | 00000000 |
| 1 | 1a: Multiplier0 = 1 → Prod = Prod + Mcand | 1011 | 00001000 | 00001000 |
|  | 2: Shift left Multiplicand | 1011 | 00010000 | 00001000 |
|  | 3: Shift right Multiplier | 0101 | 00010000 | 00001000 |
| 2 | 1a: Multiplier0 = 1 → Prod = Prod + Mcand | 0101 | 00010000 | 00011000 |
|  | 2: Shift left Multiplicand | 0101 | 00100000 | 00011000 |
|  | 3: Shift right Multiplier | 0010 | 00100000 | 00011000 |
| 3 | 1: Multiplier0 = 0 → no add | 0010 | 00100000 | 00011000 |
|  | 2: Shift left Multiplicand | 0010 | 01000000 | 00011000 |
|  | 3: Shift right Multiplier | 0001 | 01000000 | 00011000 |
| 4 | 1a: Multiplier0 = 1 → Prod = Prod + Mcand | 0001 | 01000000 | 01011000 |
|  | 2: Shift left Multiplicand | 0001 | 10000000 | 01011000 |
|  | 3: Shift right Multiplier | 0000 | 10000000 | 01011000 |

| Iter | Step (refined version) | Multiplicand | Product (left ¦ right = multiplier) |
| --- | --- | --- | --- |
| 0 | Initial: Product = 0000 ¦ multiplier | 1000 | 0000 1011 |
| 1 | 1a: Prod0 = 1 → left half += Mcand | 1000 | 1000 1011 |
|  | 2: Shift right Product (carry enters MSB) | 1000 | 0100 0101 |
| 2 | 1a: Prod0 = 1 → left half += Mcand | 1000 | 1100 0101 |
|  | 2: Shift right Product (carry enters MSB) | 1000 | 0110 0010 |
| 3 | 1: Prod0 = 0 → no add | 1000 | 0110 0010 |
|  | 2: Shift right Product (carry enters MSB) | 1000 | 0011 0001 |
| 4 | 1a: Prod0 = 1 → left half += Mcand | 1000 | 1011 0001 |
|  | 2: Shift right Product (carry enters MSB) | 1000 | 0101 1000 |

Product = 0101 1000 = 88 = 8 × 11 ✓

(b)

| Iter | Step | Multiplicand | Product |
| --- | --- | --- | --- |
| 0 | Initial: Product = 0000 ¦ multiplier | 0110 | 0000 0110 |
| 1 | 1: Prod0 = 0 → no add | 0110 | 0000 0110 |
|  | 2: Shift right Product (carry enters MSB) | 0110 | 0000 0011 |
| 2 | 1a: Prod0 = 1 → left half += Mcand | 0110 | 0110 0011 |
|  | 2: Shift right Product (carry enters MSB) | 0110 | 0011 0001 |
| 3 | 1a: Prod0 = 1 → left half += Mcand | 0110 | 1001 0001 |
|  | 2: Shift right Product (carry enters MSB) | 0110 | 0100 1000 |
| 4 | 1: Prod0 = 0 → no add | 0110 | 0100 1000 |
|  | 2: Shift right Product (carry enters MSB) | 0110 | 0010 0100 |

Product = 0010 0100 = 36 = 6 × 6 ✓

(c) §2 and §3 diagrams and flowcharts; the improvement list in the EXAM-READY box; for the combined unit §5.

# Lesson 21 — IEEE 754 Floating-Point Representation

Slides 9–16 (§3.5)

## 1. Floating Point and the Standard (sl. 9–10)

Floating point represents non-integral numbers, including very small and very large ones, like scientific notation: −2.34 × 1056 and +987.02 × 109… Normalized means exactly one non-zero digit before the point (−2.34 × 1056 yes; +0.002 × 10−4 and +987.02 × 109 no). In binary: ±1.xxxxxxx2 × 2yyyy (float and double in C). IEEE Std 754-1985 was developed because representations diverged (portability of scientific code); it is now almost universally adopted, with single (32-bit) and double (64-bit) precision.

## 2. Format (sl. 11) ★

single: ¦ S 1 ¦ Exponent 8 ¦ Fraction 23 ¦ double: ¦ S 1 ¦ Exponent 11 ¦ Fraction 52 ¦

x = (−1)S × (1 + Fraction) × 2(Exponent − Bias)

Bias: single 127, double 1023

S = 0 non-negative, 1 negative.

Significand normalized: 1.0 ≤ |significand| < 2.0. The leading 1 is always there, so it is not stored (hidden bit); significand = fraction with '1.' restored.

Exponent in excess (biased) form: stored = actual + bias, so the field is unsigned.

EXAM-READY ANSWER Why a (positive) bias (2023-24 7(b), 2018-19 7(c), 2015-16 7(c), 2012-13 8(a), 2020-21 13(a)(i)): with a bias the exponent field is an unsigned number — most negative exponent 00…01, most positive 11…10. Then two FP numbers of the same sign can be compared (and sorted) with integer comparison hardware, bit pattern by bit pattern, because a larger exponent always gives a larger field. 2's-complement exponents would make negative exponents look larger. The bias 2e−1 − 1 makes the range of exponents roughly symmetric.

## 3. Ranges and Precision (sl. 12–14)

|  | Single | Double |
| --- | --- | --- |
| Reserved exponent fields | 00000000 and 11111111 | 000…00 and 111…11 |
| Smallest normalized | exp field 1 → 1 − 127 = −126; ±1.0 × 2−126 ≈ ±1.2 × 10−38 | exp 1 → −1022; ±1.0 × 2−1022 ≈ ±2.2 × 10−308 |
| Largest | exp 254 → +127; ±(2 − 2−23) × 2127 ≈ ±3.4 × 1038 | exp 2046 → +1023; ±(2 − 2−52) × 21023 ≈ ±1.8 × 10308 |
| Relative precision | 2−23 → 23 × log102 ≈ 6 decimal digits | 2−52 → ≈ 16 decimal digits |
| Smallest denormal (textbook) | 2−23 × 2−126 = 2−149 ≈ 1.4 × 10−45 | 2−1074 ≈ 4.9 × 10−324 |

## 4. Special Values (textbook — needed for infinity, NaN and denormal questions)

| Exponent field | Fraction | Meaning |
| --- | --- | --- |
| 0 | 0 | ±0 |
| 0 | ≠ 0 | Denormal: ±0.Fraction × 21 − bias (gradual underflow) |
| 1 … max − 1 | anything | Normalized ±1.Fraction × 2E − bias |
| all 1s | 0 | ±infinity: single 0 11111111 000…0 = 0x7F800000 (−∞ = 0xFF800000) |
| all 1s | ≠ 0 | NaN (not a number), e.g. 0/0, ∞ − ∞ |

## 5. Worked Examples (sl. 15–16)

Represent −0.75: −0.75 = (−1)^1 × 1.1_2 × 2^−1 S = 1, Fraction = 1000…00, Exponent = −1 + 127 = 126 = 01111110 (double: −1 + 1023 = 01111111110) single: 1 01111110 1000…00 = 0xBF400000 double: 1 01111111110 1000…00 Decode 1 10000001 01000…00: S = 1, Fraction = 0.01_2 = 0.25, Exponent = 129 x = (−1) × (1 + 0.25) × 2^(129 − 127) = −1.25 × 4 = −5.0

## 6. Recipe for a Custom Format (16-bit, 32-bit with other field sizes) ★

1.	e = exponent bits, f = fraction bits, 1 + e + f = total. Bias = 2e−1 − 1 (unless the question fixes the range).

2.	Normalized exponents: 1 − bias … (2e − 2) − bias (fields 00…0 and 11…1 reserved).

3.	Smallest normalized = 1.0 × 21 − bias; largest = (2 − 2−f) × 22^{e − 2 − bias}.

4.	Smallest denormal = 2−f × 21 − bias. Precision = 2−f ≈ f × 0.301 decimal digits.

5.	Encode: normalize to 1.xxx × 2k; field = k + bias in e bits; fraction = bits after the point, padded to f bits.

## Exam Approach

1.	Write the format box first (S | E | F with widths).

2.	Show normalization, biased exponent, fraction padding, then group in 4s → hex.

3.	Decode: split fields, check for special exponents, then (−1)S(1 + F)2E − bias.

## Practice — Lesson 21

(a) Actual — 2019-20 4(a). (b) Actual — 2022-23 (CSE 305) 8(b) and 2020-21 13(a). (c) Actual — 2018-19 5(b). (d) Actual — 2018-19 5(c), 2017-18 5(c), 2013-14 3(b). (e) Actual — 2023-24 7(b). (f) Actual — 2021-22 8(b). (g) Actual — 2022-23 (CSE 305) 8(c).

(a) 16-bit FP; largest absolute value 1.11…1 × 263, smallest 1.00…0 × 2−62. (i) Field sizes. (ii) Represent −110.112 × 21. (iii) Smallest denormalized value. (15)

(b) 32-bit FP with 1 sign bit and (i) 19 fraction bits, (ii) 24 fraction bits, the rest exponent: precision, bias, smallest and largest absolute values, smallest denormal. (8 / 16)

(c) 16-bit FP with a 4-bit exponent: represent −1.25 and 0.375. (8)

(d) Decimal value of IEEE single 0xDF807500 and of 0x7F800005; single representation of −0.75. (5 / 5 / 10)

(e) Purpose of a positive bias; smallest and largest values in single and double precision. (15)

(f) Largest n such that every integer 1, 2, …, n is representable in IEEE single precision. (14)

(g) f1 < f2 are normalized IEEE singles. Treated as unsigned integers i1, i2 — is i1 < i2? As signed integers j1, j2 — is j1 < j2? (20)

### ✔ Solution

(a) Exponents −62 … +63 = 126 normalized values + 2 reserved = 128 = 27 → 7 exponent bits, bias 63 (field 1 → −62, field 126 → +63); fraction = 16 − 1 − 7 = 8 bits (the largest has eight 1s after the point).

(i) ¦ S 1 ¦ E 7 (bias 63) ¦ F 8 ¦ (ii) −110.11_2 × 2^1 = −1.1011_2 × 2^3 → S = 1, E = 3 + 63 = 66 = 1000010, F = 10110000 1 1000010 10110000 = 1100 0010 1011 0000 = 0xC2B0 (value −13.5) (iii) smallest denormal = 0.00000001_2 × 2^(1 − 63) = 2^−8 × 2^−62 = 2^−70 ≈ 8.5 × 10^−22

(b)

|  | (i) 19-bit fraction (2022-23) | (ii) 24-bit fraction (2020-21) |
| --- | --- | --- |
| Exponent bits, bias | 32 − 1 − 19 = 12 → bias 211 − 1 = 2047 | 32 − 1 − 24 = 7 → bias 26 − 1 = 63 |
| Precision | 2−19 ≈ 1.9 × 10−6 (≈ 5.7 decimal digits) | 2−24 ≈ 6 × 10−8 (≈ 7.2 digits) |
| Smallest normalized | 1.0 × 21 − 2047 = 2−2046 | 1.0 × 2−62 |
| Largest | (2 − 2−19) × 24094 − 2047 = (2 − 2−19) × 22047 | (2 − 2−24) × 263 ≈ 1.84 × 1019 |
| Smallest denormal | 2−19 × 2−2046 = 2−2065 | 2−24 × 2−62 = 2−86 |

(c) 4-bit exponent → bias 7, fraction 16 − 1 − 4 = 11 bits.

−1.25 = −1.01_2 × 2^0 → S 1, E 0 + 7 = 0111, F 01000000000 → 1 0111 01000000000 = 0xBA00 0.375 = 1.1_2 × 2^−2 → S 0, E −2 + 7 = 0101, F 10000000000 → 0 0101 10000000000 = 0x2C00

(d)

0xDF807500 = 1101 1111 1000 0000 0111 0101 0000 0000 S = 1, E = 1011 1111 = 191 → 191 − 127 = 64, F = 000 0000 0111 0101 0000 0000 = 0x007500 x = −(1 + 0x7500 / 2^23) × 2^64 = −1.0035705566… × 2^64 ≈ −1.8513 × 10^19 0x7F800005 = 0 11111111 000 0000 0000 0000 0000 0101 → exponent all 1s, fraction ≠ 0 → NaN −0.75 (2013-14 3(b)) = 1 01111110 1000…0 = 0xBF400000 (slide 15)

(e) Bias: EXAM-READY box in §2. Values: table in §3 — single smallest normalized ±2−126 ≈ ±1.18 × 10−38, largest ±(2 − 2−23) × 2127 ≈ ±3.40 × 1038; double ±2−1022 ≈ ±2.23 × 10−308 and ±(2 − 2−52) × 21023 ≈ ±1.80 × 10308 (denormals go down to 2−149 and 2−1074).

(f) An integer k is exact when its binary form fits in the 24-bit significand (1 hidden + 23 fraction bits). Every integer up to 224 = 16,777,216 fits (224 itself is 1.0 × 224). 224 + 1 = 1.000…0001 × 224 needs 25 significant bits → it rounds to 224. So n = 2^{24} = 16,777,216 (checked numerically).

(g)

| Case | Unsigned i1 < i2? | Signed j1 < j2? |
| --- | --- | --- |
| 0 ≤ f1 < f2 | Yes — sign 0, biased exponent then fraction order = integer order | Yes — both non-negative integers, same order |
| f1 < 0 ≤ f2 | No — i1 has MSB 1, so i1 > i2 | Yes — j1 is negative, j2 non-negative |
| f1 < f2 < 0 | No — sign-magnitude: ¦f1¦ > ¦f2¦ makes i1 > i2 | No — larger magnitude bits give the larger (less negative) j: j1 > j2 |

So neither conclusion holds in general. (i) holds only when both are non-negative; (ii) fails when both are negative. Example: f1 = −3.0 (0xC0400000), f2 = −1.0 (0xBF800000): unsigned 0xC0400000 > 0xBF800000 and signed −1,069,547,520 > −1,082,130,432.

# Lesson 22 — Floating-Point Addition and the FP Adder

Slides 17–20 (§3.5; printed 19–22)

## 1. Decimal Example (sl. 17)

9.999 × 10^1 + 1.610 × 10^−1 (4 significant digits) 1. Align decimal points: shift the number with the smaller exponent → 0.016 × 10^1 2. Add significands: 9.999 × 10^1 + 0.016 × 10^1 = 10.015 × 10^1 3. Normalize & check over/underflow: 1.0015 × 10^2 4. Round and renormalize if necessary: 1.002 × 10^2

## 2. Binary Example (sl. 18)

1.000_2 × 2^−1 + −1.110_2 × 2^−2 (0.5 + −0.4375) 1. Align: −1.110 × 2^−2 → −0.111 × 2^−1 2. Add: 1.000 × 2^−1 + −0.111 × 2^−1 = 0.001 × 2^−1 3. Normalize: 1.000 × 2^−4, with no over/underflow (−126 ≤ −4 ≤ 127) 4. Round: 1.000 × 2^−4 (no change) = 0.0625

## 3. Flowchart and Exception Conditions ★

Start 1. Compare exponents; shift the significand of the smaller number right until the exponents match 2. Add the significands (with signs) 3. Normalize: shift the sum right & increment exponent, or shift left & decrement exponent → Overflow or underflow? ── Yes → exception (∞ / denormal or 0) 4. Round the significand to the available bits → Still normalized? ── No → back to step 3 Done

| Condition | When (single precision) |
| --- | --- |
| Overflow | after normalizing, the exponent is > 127 (field would be 255) → result ±∞ |
| Underflow | after normalizing, the exponent is < −126 → denormal or 0 |
| Infinity result | an operand is ±∞ (∞ + x = ∞), or overflow |
| NaN result | an operand is NaN, or (+∞) + (−∞) |

## 4. FP Adder Hardware (sl. 19–20)

The FP adder is much more complex than an integer adder. Doing it in one clock cycle would take too long (much longer than integer operations) and a slower clock would penalize all instructions, so the FP adder usually takes several cycles and can be pipelined.

Figure 22.1 — FP adder: small ALU (exponent difference), shift right, big ALU, normalize, round (sl. 20)

Step 1: the small ALU subtracts the exponents; the difference controls the muxes and the shift-right unit that aligns the smaller significand.

Step 2: the big ALU adds the significands.

Step 3: normalize — the leading-zero logic shifts left or right and the increment/decrement unit adjusts the exponent.

Step 4: rounding hardware; if rounding de-normalizes the result, go back to step 3.

## Exam Approach

1.	Convert both numbers to normalized binary (enough fraction bits).

2.	Align the smaller exponent to the larger one by shifting its significand right.

3.	Add with signs, normalize, check the exponent range, round to the format's fraction bits.

4.	Give the final bit pattern and its decimal value as a check.

## Practice — Lesson 22

(a) Actual — 2016-17 7(b). (b) Actual — 2017-18 5(b). (c) Actual — 2022-23 1(a), 2018-19 8(b), 2015-16 6(a), 2013-14 3(a), 2011-12 5(a).

(a) Using the FP adder hardware, simulate −(0.25) + (0.625) by filling: binary, step 1 exponent matching, step 2 significand addition, step 3 normalization, step 4 rounding. (15)

(b) 16-bit half precision with a 4-bit exponent: convert 0.75 and 1.25 and add them step by step, giving the exponent and fraction fields at every step. (15)

(c) Steps of FP addition (flowchart) with an example; overflow, underflow, NaN and infinity conditions. (10–12)

### ✔ Solution

(a)

|  | −(0.25)10 | (0.625)10 |
| --- | --- | --- |
| Binary | −1.000 × 2−2 (1 01111101 000…0) | 1.010 × 2−1 (0 01111110 010…0) |
| Step 1: exponent matching | −0.100 × 2−1 (shift right 1, exp −2 → −1) | 1.010 × 2−1 (unchanged) |
| Step 2: significand addition | 1.010 + (−0.100) = 0.110 × 2−1 |  |
| Step 3: normalization | 0.110 × 2−1 = 1.100 × 2−2; −126 ≤ −2 ≤ 127 → no over/underflow |  |
| Step 4: rounding | 1.100 × 2−2 (no change) = 0 01111101 1000…0 = 0x3EC00000 = 0.375 |  |

(b) Bias 7, 11 fraction bits.

0.75 = 1.1_2 × 2^−1 → S 0, E 0110 (6), F 10000000000 = 0x3400 1.25 = 1.01_2 × 2^0 → S 0, E 0111 (7), F 01000000000 = 0x3A00 Step 1 (align): exponents 6 < 7 → shift 0.75: 1.10000000000 × 2^−1 → 0.11000000000 × 2^0, E = 0111 Step 2 (add): 1.01000000000 + 0.11000000000 = 10.00000000000 × 2^0 Step 3 (normalize): 1.00000000000 × 2^1 → E = 1 + 7 = 1000; 1 ≤ 8 ≤ 14 → no overflow/underflow Step 4 (round): no bits were lost → F = 00000000000 Result: 0 1000 00000000000 = 0x4000 = 2.0 ✓

(c) Flowchart and condition table in §3 with the slide-18 binary example (0.5 + −0.4375 = 0.0625).

# Lesson 23 — Beyond the Short Slides (textbook insurance)

Not on Tushar Sir's short slides — P&H §3.3–3.6. Asked: division ×4, MIPS FP code ×4, signed multiplication, guard/round/sticky, subword parallelism

NOTE These questions are in the tracker's Outside Notes tab because the short slides do not cover them. Division and FP code were asked as recently as 2022-23 (CSE 209 1(b), CSE 305 8(a)) — ask sir whether they are in this term's syllabus.

## 1. Division Hardware and Algorithm (§3.4)

First version: 64-bit Divisor register (divisor starts in the LEFT half) shifting right, 64-bit ALU, 64-bit Remainder register (initially the dividend), 32-bit Quotient shifting left. 33 repetitions:

1. Remainder = Remainder − Divisor 2. Test Remainder: ≥ 0 → shift Quotient left, set new rightmost bit 1 < 0 → restore: Remainder = Remainder + Divisor; shift Quotient left, new bit 0 3. Shift Divisor right 1 bit; 33rd repetition? No → step 1, Yes → Done

Refined version (the hardware of 2012-13 8(b)): 32-bit Divisor, 32-bit ALU, 64-bit Remainder register that shifts left; the quotient bits enter its right half. Algorithm: shift Remainder left 1; repeat n times {left half − Divisor; if ≥ 0 keep it and shift left with new bit 1, else restore and shift left with new bit 0}; finally shift the left half right 1 bit. Then left half = remainder, right half = quotient. It needs the dividend's upper half < divisor (otherwise the quotient overflows n bits).

### 2022-23 1(b) ★ — 6-bit ALU: 1011 1100 1111 ÷ 110101 (3023 ÷ 53)

| Iter | Step (refined, 6-bit ALU) | Divisor | Remainder register (left ¦ right) |
| --- | --- | --- | --- |
| 0 | Initial: Rem = dividend | 110101 | 101111 ¦ 001111 |
|  | Shift Rem left 1 | 110101 | 1011110 ¦ 011110 |
| 1 | Left half − Div ≥ 0 → keep; shift Rem left, R0 = 1 | 110101 | 1010010 ¦ 111101 |
| 2 | Left half − Div ≥ 0 → keep; shift Rem left, R0 = 1 | 110101 | 0111011 ¦ 111011 |
| 3 | Left half − Div ≥ 0 → keep; shift Rem left, R0 = 1 | 110101 | 0001101 ¦ 110111 |
| 4 | Left half − Div < 0 → restore; shift Rem left, R0 = 0 | 110101 | 0011011 ¦ 101110 |
| 5 | Left half − Div < 0 → restore; shift Rem left, R0 = 0 | 110101 | 0110111 ¦ 011100 |
| 6 | Left half − Div ≥ 0 → keep; shift Rem left, R0 = 1 | 110101 | 0000100 ¦ 111001 |
|  | Shift left half right 1 (final fix-up) | 110101 | 000010 111001 |

Quotient = 111001 = 57, Remainder = 000010 = 2   (57 × 53 + 2 = 3023 ✓)

### 2018-19 6(b) — 4-bit: 11001001 ÷ 1011 (201 ÷ 11), first version

| Iter | Step | Quotient | Divisor | Remainder |
| --- | --- | --- | --- | --- |
| 0 | Initial values | 00000 | 10110000 | 11001001 |
| 1 | 1: Rem = Rem − Div → Rem ≥ 0 | 00000 | 10110000 | 00011001 |
|  | 2a: Shift Q left, Q0 = 1 | 00001 | 10110000 | 00011001 |
|  | 3: Shift Divisor right | 00001 | 01011000 | 00011001 |
| 2 | 1: Rem = Rem − Div → Rem < 0 | 00001 | 01011000 | 11000001 |
|  | 2b: Rem = Rem + Div (restore); shift Q left, Q0 = 0 | 00010 | 01011000 | 00011001 |
|  | 3: Shift Divisor right | 00010 | 00101100 | 00011001 |
| 3 | 1: Rem = Rem − Div → Rem < 0 | 00010 | 00101100 | 11101101 |
|  | 2b: Rem = Rem + Div (restore); shift Q left, Q0 = 0 | 00100 | 00101100 | 00011001 |
|  | 3: Shift Divisor right | 00100 | 00010110 | 00011001 |
| 4 | 1: Rem = Rem − Div → Rem ≥ 0 | 00100 | 00010110 | 00000011 |
|  | 2a: Shift Q left, Q0 = 1 | 01001 | 00010110 | 00000011 |
|  | 3: Shift Divisor right | 01001 | 00001011 | 00000011 |
| 5 | 1: Rem = Rem − Div → Rem < 0 | 01001 | 00001011 | 11111000 |
|  | 2b: Rem = Rem + Div (restore); shift Q left, Q0 = 0 | 10010 | 00001011 | 00000011 |
|  | 3: Shift Divisor right | 10010 | 00000101 | 00000011 |

Quotient = 10010 = 18, Remainder = 0011 = 3   (18 × 11 + 3 = 201 ✓)

Note: the dividend's upper half 1100 ≥ divisor 1011, so the quotient needs 5 bits — with the refined 4-bit hardware this is a divide overflow; the first version (5 iterations) handles it.

### 2017-18 8(a) — 11010101 ÷ 1011 (213 ÷ 11), first version

| Iter | Step | Quotient | Divisor | Remainder |
| --- | --- | --- | --- | --- |
| 0 | Initial values | 00000 | 10110000 | 11010101 |
| 1 | 1: Rem = Rem − Div → Rem ≥ 0 | 00000 | 10110000 | 00100101 |
|  | 2a: Shift Q left, Q0 = 1 | 00001 | 10110000 | 00100101 |
|  | 3: Shift Divisor right | 00001 | 01011000 | 00100101 |
| 2 | 1: Rem = Rem − Div → Rem < 0 | 00001 | 01011000 | 11001101 |
|  | 2b: Rem = Rem + Div (restore); shift Q left, Q0 = 0 | 00010 | 01011000 | 00100101 |
|  | 3: Shift Divisor right | 00010 | 00101100 | 00100101 |
| 3 | 1: Rem = Rem − Div → Rem < 0 | 00010 | 00101100 | 11111001 |
|  | 2b: Rem = Rem + Div (restore); shift Q left, Q0 = 0 | 00100 | 00101100 | 00100101 |
|  | 3: Shift Divisor right | 00100 | 00010110 | 00100101 |
| 4 | 1: Rem = Rem − Div → Rem ≥ 0 | 00100 | 00010110 | 00001111 |
|  | 2a: Shift Q left, Q0 = 1 | 01001 | 00010110 | 00001111 |
|  | 3: Shift Divisor right | 01001 | 00001011 | 00001111 |
| 5 | 1: Rem = Rem − Div → Rem ≥ 0 | 01001 | 00001011 | 00000100 |
|  | 2a: Shift Q left, Q0 = 1 | 10011 | 00001011 | 00000100 |
|  | 3: Shift Divisor right | 10011 | 00000101 | 00000100 |

Quotient = 10011 = 19, Remainder = 0100 = 4   (19 × 11 + 4 = 213 ✓)

## 2. Signed Multiplication (2020-21 10(c))

Simplest method: remember the signs, convert both operands to positive (2's-complement negate the negative ones), multiply the magnitudes with the unsigned algorithm for 31 iterations, and negate the product if the original signs differed (sign = SA ⊕ SB). Alternatively, sign-extend the partial products and treat the last step as a subtraction (Booth's algorithm: look at multiplier bits (bi, bi−1): 10 → subtract multiplicand, 01 → add, 00/11 → only shift, using arithmetic right shifts) — this works directly on 2's-complement numbers.

## 3. MIPS Floating-Point Instructions and Code (§3.5)

Separate FP registers $f0–$f31; a double uses an even/odd pair ($f0/$f1 …).

Arithmetic: add.s / add.d, sub.s / sub.d, mul.s / mul.d, div.s / div.d. Loads/stores: lwc1 / swc1 (single), ldc1 / sdc1 (double).

Compare: c.xx.s / c.xx.d (xx = eq, lt, le) sets a condition bit; branch with bc1t (true) / bc1f (false).

### 2022-23 (CSE 305) 8(a) = 2020-21 13(c) ★: float a, b, c; double w, x, y, z; if (a + b > c) x = (y*z)/w;

# a $f0, b $f1, c $f2 (single); w $f4, x $f6, y $f8, z $f10 (double pairs) add.s $f3, $f0, $f1 # f3 = a + b c.le.s $f3, $f2 # (a + b) <= c ? bc1t Skip # yes → condition false, skip mul.d $f12, $f8, $f10 # y * z div.d $f6, $f12, $f4 # x = (y * z) / w Skip:

### 2019-20 4(c) ★: double; if (a > b) c = c − (c*e)/d; else c = c + d;  (a $f2, b $f4, c $f6, d $f8, e $f10)

c.le.d $f2, $f4 # a <= b ? bc1t Else mul.d $f12, $f6, $f10 # c * e div.d $f12, $f12, $f8 # (c * e) / d sub.d $f6, $f6, $f12 # c = c - (c*e)/d j Exit Else: add.d $f6, $f6, $f8 # c = c + d Exit:

### 2017-18 8(b): float convert(float cels) { return 9.0*cels/5.0 + 32.0; }  (cels in $f12, result in $f0)

convert: lwc1 $f16, const9($gp) # 9.0 from static data lwc1 $f18, const5($gp) # 5.0 mul.s $f16, $f16, $f12 # 9.0 * cels div.s $f16, $f16, $f18 # / 5.0 lwc1 $f18, const32($gp) # 32.0 add.s $f0, $f16, $f18 # + 32.0 jr $ra

## 4. Guard, Round and Sticky Bits (2020-21 12(c))

During alignment and normalization, bits shifted out of the significand would be lost. IEEE 754 keeps two extra bits on the right during the operation — guard and round — so that the result can be rounded as if it had been computed exactly and then rounded. Example with 3 significant decimal digits: 2.56 × 100 + 2.34 × 102: with guard/round digits 2.3400 + 0.0256 = 2.3656 → rounds to 2.37; without them 2.34 + 0.02 = 2.36 (an error of 1 in the last digit). The sticky bit is set if any nonzero bit was shifted beyond the round bit; it tells round-to-nearest-even whether the discarded part is exactly half (…100…0) or more than half, so ties are broken correctly. Together they give results accurate to ½ unit in the last place.

## 5. Subword Parallelism (2021-22 5(c))

A wide (64/128-bit) adder can be partitioned so that one instruction operates on many short operands at once: 16 × 8-bit, 8 × 16-bit, 4 × 32-bit or 2 × 64-bit (SIMD — MMX/SSE/AVX, ARM NEON). Image processing works on 8-bit pixels, so 16 pixels are added, averaged or saturated in one instruction; matrix multiplication uses packed multiply-add on 4 floats/doubles at a time, turning the inner loop into one quarter (or less) of the scalar instructions with the same hardware.

# Quick Notes — Arithmetic

Revision points for Lessons 19–23.

## L19 · Add/sub overflow

Add overflow: (+)+(+) → sign 1; (−)+(−) → sign 0. Subtract: (+)−(−) → 1; (−)−(+) → 0. Never for opposite-sign add / same-sign subtract.

V = O'SA'SB'SR + O'SASBSR' + OSA'SBSR + OSASB'SR'.

## L20 · Multiplication

First: 64-bit multiplicand ←, 32-bit multiplier →, 64-bit ALU & product; add, shift, shift × 32.

Refined: 32-bit multiplicand & ALU, multiplier in right half of 64-bit product, add then shift product → (1 cycle/step).

HI/LO; mult/multu → HI:LO; mul → low 32 bits. Combined mult/div: same refined datapath, product register shifts both ways.

## L21 · IEEE 754

x = (−1)S(1 + F)2E − bias; single 1/8/23 bias 127; double 1/11/52 bias 1023.

Single 2−126 … (2 − 2−23)2127; denormal 2−149. ∞: E all 1s, F = 0 (0x7F800000). NaN: E all 1s, F ≠ 0.

Bias → unsigned exponent → integer comparison works. Custom: bias 2e−1 − 1; min 21−bias; max (2 − 2−f)22^{e−2−bias}; denorm 2−f·21−bias.

−0.75 = 0xBF400000; 0xC0A00000 = −5.0; 0xDF807500 ≈ −1.85 × 1019; integers exact up to 224.

## L22 · FP addition

Align (shift smaller right) → add → normalize + check over/underflow → round (→ renormalize).

−0.25 + 0.625 = 1.1 × 2−2 = 0.375; half precision 0.75 + 1.25 = 0x4000 = 2.0.

## L23 · Textbook extras

Division: subtract, test sign, restore or set 1, shift; refined: remainder register shifts left, quotient enters right half; final left-half shift right.

3023/53 = 57 r 2; 201/11 = 18 r 3; 213/11 = 19 r 4.

FP code: add.s/d, mul, div, c.le.s + bc1t; doubles in even registers.

Guard + round + sticky → correct rounding; subword parallelism = partitioned adder (SIMD).

## Key Diagrams to Reproduce

Optimized multiplier (sl. 7) — same as Fig. 20.2

FP adder hardware (sl. 20) — same as Fig. 22.1